_________________________________

VTK™ for Newton

User's Guide

Preliminary 3/3/95

©1995 Abbate Video, Inc.

CONTENTS

1	WELCOME!

Unpacking

Requirements

2	CONNECTING

Identify Your Control Ports

Connect AV/net to the Control Port

Connect the Newton to AV/net

3	INSTALLING THE SOFTWARE

Load the Software

Launch VTK

Confirm VCR Type

Test VCR Control

4	USING VTK FOR NEWTON

The Control View

The Notes Format

Time of Day Logging

5	REFERENCE INFORMATION

About VTK for Newton

About AV/net

Accuracy and Timecode

Troubleshooting

Equipment List

Contacting Abbate

Chapter 1

WELCOME

Welcome to Abbate Video's VTK for Newton! With this package, you can control popular consumer and professional VCRs right from your Newton.

Just load the software, connect the control module, and control and log your footage the easy way!

VTK for Newton is more than a simple IR remote replacement. VTK for Newton provides two way communication with the camcorder or VCR.  This two way connection lets you create a time log of counter or timecode numbers with just a tap on the Newton.

Furthermore, VTK for Newton seamlessly integrates into the Newton environment. The logged time readings are placed directly into the Notes "soup" in a straightforward comma delimited format.

UNPACKING

Your package contains:

•	Software Diskette(s)

•	AV/net Control Module

•	User's Guide

•	Registration Form (bound into the center of this guide)

Please take a moment now to remove and fill out your Registration Form, then mail it back to us using the envelope we've enclosed. We're also interested in any initial comments you might have, so please make any notes on the form as well.

REQUIREMENTS

To use VTK for Newton, you'll need the following:

Newton Message Pad OMP. MP100.MP110,MP120,ExpertPad

Controllable Camcorder or VCR supporting one of the following remote control protocols:

Control-L (LANC)

Control-M (Panasonic 5-pin)

ViSCA

RS-422

RS-232

Connection Kit or other software to install the VTK for Newton package.

Chapter 2

CONNECTING

VTK for Newton controls camcorders and VCRs through Abbate's AV/net device control modules. An interface cable connects the Newton serial port to the AV/net module.  The AV/net module connects directly to the remote control port on your camcorder or VCR.

Identify Your Control Ports

Take a moment to locate and identify the control port on your camcorder, VCR or VTR.

The most common control port on consumer camcorders is a Control-L or LANC port. It is usually labeled "Remote" and is sometimes covered by a plastic dust plug.

Some VCRs and older camcorders have a 5-pin Control-L or LANC port: Your AV/net module may include both the sub-mini and 5 pin mini-din styles of Control-L connectors. Alternatively, the AV/net module may have only the sub-min connector. In this case you will need the sub-mini to 5 pin adapter. Contact Abbate.

IMPORTANT:

Control-L or LANC remotes found on VCRs can operate in one of two modes: Master or Slave mode. To be controlled from VTK the mode MUST BE SLAVE.

Some VCRs have an actual physical switch on the deck to set the Control-L mode. Often the switch is labeled M/S for Master/Slave.

Other VCRs use an on-screen menu-driven switch to set the mode. The setting is usually called LANC Mode and the choices are M/S.

Sony's Vdeck and Vbox use the 8-pin mimi-din round serial connector.

The Panasonic AG-1960/1970 and FS-88/100/200/1000 have a special 5 pin Control-M port.

Professional RS-422 VTRs as well as some RS-232 VCRs have a standard 9-pin "D" serial connector.

Some RS-232 VCRs such as the Panasonic AG-5700 and Mitsubishi BV-2000 have a standard 25-pin "D" serial connector.

IMPORTANT:

These VCRs are equipped with a Remote/Local switch, usually located on the front panel. The mode must be set to Remote to allow VTK to control the VCR.

Connect AV/net to the Control Port

After you have identified your control port and matching control cable, simply connect the cable that is part of the AV/net module to the remote port on your camcorder, VCR or VTR.

IMPORTANT

The AV/net module is powered from the camcorder, VCR or VTR.

Connect the Newton to AV/net

Connect the 8 pin mini-din connector to the Newton serial port.  Connect the RJ11 plug to the "in" connector on the AV/net module. The "In" connector is on the left side of the module when the module is sitting on the flat side.

Chapter 3

INSTALLING THE SOFTWARE

Follow the instructions in this chapter to install the VTK for Newton software.

Load the Software

Insert the VTK for Newton Disk in your floppy drive. You will need the Newton Connection Kit or other third party utility to upload the VTK for Newton Package to your Newton. Follow the procedure described in the uploader documentation.  The file to upload is "VTK for Newton.pkg" for mac or "VTKNEWT.PKG" for PC.

Be sure to read the README file for the latest release notes.

Launch VTK

Go to "Extras" on the Newton and tap "VTK". The VTK for Newton application will appear.  The application floats on top of other applications and can be dragged to anywhere on the screen.

Close other applications so that the standard Newton Notes view is visible behind VTK.  VTK stores logged time numbers in the Notes "soup".

Confirm VCR Type

If the Newton is connected to an active AV/net module the application will read the AV/net module type and display the type on the screen. If an AV/net module is not connected "AV/net N/A" will be displayed.

Test VCR Control

Tap the transport control buttons and confirm proper VCR control.

Chapter 4

USING VTK FOR NEWTON

This chapter describes the use of VTK for Newton. The application can be used with or without the AV/net interface, we'll describe both configurations here..

The Control View

The main view provides the following functions:

Transport Control

Time Logging

Shot Comments

AV/net Feedback

The transport control buttons operate in the same way as the buttons on the VCR or on a handheld IR remote.

>>>	Fast Forward (from Stop mode)

<<<	Rewind (from Stop mode)

>>	Scan Forward (from Play or Pause mode, a tap latches scan)

<<	Scan Reverse (from Play or Pause mode, a tap latches scan)

>	Play

||	Pause (Toggle)

[]	Stop

"Set In" and "Set Out" queries the current time from the device and place the time number directly into the standard Newton Notes area. See below for the format VTK uses.

"Shot" provides a pop-up picker which will place the selected item directly into the standard Newton Notes area.

The field on the bottom displays the AV/net status on startup and confirms proper AV/net command reception by displaying "OK".

The Notes Format

VTK for Newton places time and comment information in the currently active Note in the following format:

1, hh:mm:ss:ff,, hh:mm:ss:ff,

2, hh:mm:ss:ff,, hh:mm:ss:ff,

3, hh:mm:ss:ff, comment, hh:mm:ss:ff,

n, hh:mm:ss:ff, comment, hh:mm:ss:ff,more comments

The exported comma delimited fields are:

eventNumber,InTime,Comment1,OutTime,Comment2

Tapping "Set In" creates a new paragraph.  An event number is placed at the start of that line, followed by a comma and a space. The time is read from the VCR and is placed in hh:mm:ss:ff form, followed by another comma.

A comment can be written in, or added from the shot picker.

Tapping "Set Out" adds a comma followed by a space. The time is read from the VCR and is placed in hh:mm:ss:ff form, followed by another comma.

Another comment can be written in, or added from the shot picker.

The event numbers reset to 1 when a new note is created and scrolled to the top of the screen.  We suggest that you treat one note as one scene. A new note then function as a scene delimiter.  Remember that the "active" note is the note at the top of the screen.

Time of Day Logging

VTK for Newton will log the time of day in hh:mm:ss:ff form when not connected to an active AV/net module.

Chapter 5

REFERENCE INFORMATION

This chapter contains useful reference information, helpful tips and a supported equipment list

About VTK for Newton

VTK for Newton is based on the same technologies found in Abbate Video's VideoToolkit family of software / hardware products.

The Newton and a controllable VCR, connected by VTK for Newton, form a simple, powerful and inexpensive logging system

The logging source device must have remote control port capable of two-way communication (like a data bus). This is because VTK for Newton must be able to both send commands to the camcorder or VCR, as well as receive status information back from it (like time code or tape counter location data).

Examples of two-way control ports include Control-L (LANC) (found on most Hi-8 and 8 mm camcorders and VCRs, and some Sony S-VHS and VHS VCRs), ViSCA (found on the Sony Vdeck and Vbox), Control-M (found on Panasonic prosumer VCRs), RS-422 (found on professional decks), and RS-232 (found on professional and "prosumer" decks).

In all cases, you will control your source by connecting an AV/net module to its two-way control port.

About AV/net™

See the accompanying specifications for more detailed information about AV/net.

Accuracy and Timecode

Typical accuracy when used with RC timecode is +/- 1 frames. Typical accuracy when used with LTC/VITC timecode is +/- 0 frames. Otherwise, accuracy depends on tape counter/transport accuracy of controlled device.

Troubleshooting

This section describes a typical troubleshooting session.

First confirm that the Newton to AV/net cable is connected to the "In" jack on the AV/net module.

Next, confirm that the AV/net module connector is plugged into the correct remote jack on the camcorder or VCR.

Tap the VTK for Newton application and confirm that the correct AV/net module type is displayed.

Finally, confirm proper transport control and time reading.

About AV/net Initialization

The AV/net module is powered by the camcorder or VCR that it is connected to.  When first powered on the AV/net module is not initialized. Before the AV/net module can control the device it must be initialized.

The VTK for Newton software performs this initialization when it starts up.  The results of the initialization are displayed at the bottom of the VTK view. If successful, the particular node type, LANC, M etc., will be displayed.  If not successful N/A (not available) will be displayed.

IMPORTANT

If the camcorder or VCR is powered off during a session the AV/net module will not be initialized when the camcorder or VCR is powered on.

Simply close the VTK for Newton application, then restart the application.

About Time Reading

If there is any question about the accuracy fo the reported time numbers, then you should verify timecode and VCR configuration.

VTK can log based on the counter or timecode information on the tape.

If VTK is set to request Counter information make sure that the tape does not have unrecorded sections of tape, especially gaps that are near marked scenes. Most current VCR counters run when there is recorded video, but stop when there is not recorded video.

If VTK is set to request Counter make sure that the tape has been rezeroed before logging.

If VTK is set to request either RC, LTC or VITC timecode make sure that the code on the tape is continuous and ascending.

RC:

VCRs or camcorders that record RC code usually work as follows:

If recording begins on a blank section of tape the RC code is set to 00:00:00:00. If recording begins over existing video the RC code picks up

from the number that is on that section of tape. It is important that the tape NOT have code that counts upwards but then resets to zero in the middle.

VCRs or camcorders that read RC code can be set to display and report either counter or RC information. The time display on the VCR usually indicates the mode that is selected. If Counter, H:MM:SS data will be displayed. If RC, H:MM:SS:FF data will be displayed.

LTC or VITC:

Professional VCRs that are equipped with timecode readers can be set for different record modes. Consult the operation guide for timecode setup information. The goal is to verify that the selected timecode is continuous and ascending on tape.

Professional VCRs can display one location on the front display but report a different location through the remote. If VTK is configured to request LTC but the VCR display is set to display Counter the numbers will probably not agree. Consult the operation guide for your VCR for instructions on setting the display mode.

About control L or remote settings

VCRs equipped with Control-L (LANC) can run in 2 modes: Master or Slave.

THE VCR MUST BE SET TO SLAVE MODE to be controlled by VTK.

VCRs have one of 2 methods for choosing the mode. Some models have a physical switch labeled M/S or Master/Slave. The switch is sometimes located near the LANC connector, in other cases it is located on the front panel. Some models have an internal on-screen menu selection. Typically it is identified as LANC Mode with M/S choices.

If VTK still appears to not behave properly please contact us. Before calling have the results from the above tests ready. Also, have your serial number available, as well as the model number of the VCR you are controlling..

Equipment List

This is the list of camcorders and VCRs that implement standard control ports at the time of publication. These camcorders and VCRs are listed by protocol and manufacturer.

We believe that VTK for Newton will properly control any device with the standard control ports listed below., and we have listed for reference only those manufacturer's models which we beleive have these control ports. Be sure to confirm that any equipment you intend to buy or use with the

VTK for Newton has the necessary control port by checking with your dealer, the equipment manufacturer, or the equipment Owner's Manual.

VTK for Newton should work properly with nearly any camcorder or VCR supporting one of the following two-way remote protocols:

Control-L (LANC)

Control-M (selected Panasonic 5-pin)

ViSCA

RS-422

RS-232

Although we have tested VTK for Newton extensively, we cannot be responsible for changes that manufacturers make to their camcorders or VCRs that may affect the operation of the VTK for Newton

The following models contain a wired remote port:

Canon

Al                		Hi-8 Camcorder

Al/Digital   		Hi-8 Camcorder

Al/MKII      		Hi-8 Camcorder

LI PRO        		Hi-8 Camcorder

L2 PRO       		Hi-8 Camcorder w/RC

Nikon

VN-500       		8mm Camcorder

VN-950/960   	Hi-8 Camcorder

Ricoh

R-66            		8mm Camcorder

R-86S          		8mm Camcorder

R-67            		8mm Camcorder

R-88          	 	Hi-8 Camcorder

R-105S         	8mm Camcorder

R-108H         	Hi-8 Camcorder w/RC

R-800           	Hi-8 Camcorder

R-830/831   	8mm Camcorder

R-861          		8mm Camcorder

Sony VCRs

EVA-50           	8mm Player/Recorder

EVC-3                         8mm Player/Recorder

EVC-8U (*)    	8mm Player/Recorder

EVC-40  		8mm Player/Recorder

EVC-100      		Hi-8 Player/Recorder

EVA-50          	8mm Player/Recorder

EVS-550       		8mm Player/Recorder

EV-DT2             	8mm Player/Recorder/Monitor

EVO-9100        	Hi-8 Camcorder w/Hi-8TC

EVS-550/800     	8mm Player/Recorder

EVS-900/3000  	Hi-8 Player/Recorder

EVS-1000		PAL Hi-8 Player/Recorder

EVS-7000		Hi-8 Player/Recorder w/RC

EVS-9000		PAL Hi-8 Player/Recorder w/RC

GV-500               	Walkman

GVS-50                 	Walkman

SLV-30/50/70     	VHS Player/Recorder

SLV-676/757       	VHS Player/Recorder

SVO-160                	VHS Player/Recorder

SLV-R5             	S-VHS Player/Recorder

SLV-1000          	S-VHS Player/Recorder

Visca

CI-1000              	V-Box Computer Interface

CVD-1000		Hi-8 Player/Recorder w/RC

CVD-500		8mm Player/Recorder w/RC

Betamax

Assorted L equipped models

Sony Camcorders

CCD-F33/35		8mm Camcorders

CCD-F40/50/55     	8mm Camcorders

CCD-70              	8mm Camcorders

CCD-V9/11    	8mm Camcorders

CCD-V220      	8mm Camcorder

CCD-V201/301	8mm Camcorders

CCD-V401/501  	8mm Camcorders

CCD-V99/101/701 	Hi-8 Camcorders

CCD-V801       	Hi-8 Camcorder  w/RC

CCD-V5000       	Hi-8 Camcorder

CCD-V6000   	PAL Hi-8 Camcorder w/RC

CCD-SP9      		8mm Camcorder

FX-310/410/510	8mm Camcorder

FX-710         		Hi-8 Camcorder

CCD-TR4/5/6/7 	8mm Camcorder

CCD-TR51              	8mm Camcorder

CCD-TR71              	8mm Camcorder

TR81/101/200       	Hi-8 Camcorder

TR700         	 	Hi-8 Camcorder w/RC

CCD-VX3                   	Hi-8 Camcorder w/RC

Yashica

KDH-170        	8mm Camcorder

KDM-770        	8mm Camcorder

KX-70/90       	8mm Camcorder

KX-77          		8mm Camcorder

KD-H1          		Hi-8 Camcorder

KDM-170        	Hi-8 Camcorder w/RC Time Code

Panasonic Control-M

AG-1950/1830/1960/1970	S-VHS

FS-88/100/200			PAL S-VHS

FS-1000				PAL S-VHS w/VITC option

RS422 Protocol

Sony

BVU 8xx/9xx		U-Matic

VO-9800/9850	U-Matic

BVW			Betacam

PVW			Betacam

UVW 1600/1800	Betacam

EVO-9800/9850	Hi8

Panasonic

AG-7650/7750	S-VHS Feeder/Editor

AG-850		S-VHS Editor

JVC

BR-S822U      	S-VHS Editor

BR-S622U      	S-VHS Source

Edit Desk Family	S-VHS

RS232 Protocol

Mitsubishi

BV-2000 		S-VHS Player/Recorder

Panasonic

AG-5700        	S-VHS Player/Recorder

TQ-3031		Writeable Optical

Sanyo

GVR-S950        	S-VHS Player/Recorder w/LTC

Contacting Abbate

Abbate Video, Inc.

14 Ross Ave, Floor 3

Millis, MA 02054

Telephone (508) 376-3712   Fax (508) 376-3714

InterNet: mpa@abbate.com or ftp.abbate.com

AppleLink: "Abbate.Video"

AOL: "Abbate" or "Abbate Vid"

Online Support: America OnLine: Keyword   "Abbate"

VTK for Newton Model Numbers

AV-705	VTK for Newton, Sony LANC

SOURCE:	LANC (Control-L) camcorder or VCR

AV-710	VTK for Newton, Panasonic Control-M

SOURCE:	AG1960/1970 FS88/100/200/1000

AV-706	VTK for Newton, Visca

SOURCE:	Visca Vbox or VCR

AV-722	VTK for Newton, RS-422

SOURCE:	RS-422 BVU/BVW/PVW/UVW1600-1800

EVO/VO-9800/9850

AG7650-7750-850/BRS622-822/EditDesk

AV-711	VTK for Newton, Panasonic RS-232

SOURCE:	AG-5700/TQ3031

AV-712	VTK for Newton, Mitsubishio RS-232

SOURCE:	BV2000

AV-714	VTK for Newton, Sanyo RS-232

SOURCE:	GVRS950