The RC5 code: -------------  The RC5 code is an 13-bit code, it used to have a 5-bits group and a 6-bits command, and 2 stop-bits, but it was changed to 5-bits group, 7-bits command and 1 stop-bit.   Carrier: 36kHz  No header.                  +----+ 1 is coded:     |    |             ----+    +              T    T              +----+ 0 is coded: |    |             +    +----               T    T  T = 889us  Old RC5: data length: 14bit (2sync bits, 1 control bit, 11 data bits) 24,889ms  Example to play video: store 5 in GROUP put 53 on stack press RC5REMOTE  ****************************************************************************** ------------------------------------------------------------------------------ Commnd:	Video mode: TV1 & TV2	Video mode: TXT		Video mode: VD 	Group 0 (TV1) 1 (TV2)	Group 2			Group 3 ------------------------------------------------------------------------------   0	0			0			0   1	1			1			1   2	2			2			2   3	3			3			3   4	4			4			4   5	5			5			5   6	6			6			6   7	7			7			7   8	8			8			8   9	9			9			9  10	1/2/3 digs (japan:10)	step page +		=/= =/=  11	channel/program(jap:11)	step page -		*  12	standby			standby			standby  13	mute/demute		mute/demute		mute/demute  14	P.P. (personal pref.)	P.P.			P.P.  15	display			display			display  16	volume +		volume +		volume +  17	volume -		volume -		volume -  18	brightness +		brightness +		brightness +  19	brightness -		brightness -		brightness -  20	colour saturation +	colour saturation +	colour saturation +  21	colour saturation -	colour saturation -	colour saturation -  22	bass +			bass +			bass +  23	bass -			bass -			bass -  24	treble +		treble +		treble +  25	treble -		treble -		treble -  26	balance right		balance right		balance right  27	balance left		balance left		balance left  28	contrast +		enter/validation (Frnc)  29	contrast -		memory out  30	search +		sequence out		attribute on/off  31	tint/hue +  32	channel/program +	exchange  33	channel/program -	index/summary (France)	index  34	alt/channel		row zero (France)  35	1st/2nd language	1st/2nd language	1st/2nd language  36	spacial stereo on/off	spacial stereo on/off	spacial stereo on/off  37	mono/stereo (USA)  38	sleeptimer		hardcopy printer	hardcopy printer  39	tint/hue -		antiope TV mixed (Frnc)  40	RF switch/PIP select	PIP select		terminal initiation  41	vote			page hold on/off  42	timed page/channel clck	timed page (F:magazine)  43	increment (USA)		large top/lrge bot/norm	large top/lrge bot/norm  44	decrement (USA)		reveal/conceal/canc pic  45				cancel teletext picture  46	2nd controls (USA:menu)	TV/Text mixed/Subtitle	TV/Text mixed  47	PIP shift		PIP shift		view data line off  48	pause			pause			pause  49	erase			erase			erase  50	rewind			rewind			rewind  51	go to			go to			go to  52	wind

wind			wind  53	play			play			play  54	stop			stop			stop  55	recording		recording		recording  56	external 1		external 1		external 1  57	external 2		external 2		external 2  58	PIP display mode	PIP display mode  59	view data mode/advance				view data line on  60	teletext submode (J:12)  61	system standby		system standby		system standby  62	crispener on/off	news flash (France)  63	system select		system select		system select  64	surround sound		surround sound		surround sound  65	balance front		balance front		balance front  66	balance rear		balance rear		balance rear  67	sound effects		sound effects		sound effects  68	sound effects		sound effects		sound effects  69	sound effects		sound effects		sound effects  70	sound effects		sound effects		sound effects  71	sound effects		sound effects		sound effects  72	sound or menu func.	sound or menu func.	sound or menu func.  73	sound or menu func.	sound or menu func.	sound or menu func.  74	sound or menu func.	sound or menu func.	sound or menu func.  75  76  77	linear function up	linear function up	linear function up  78	linear function down	linear function down	linear function down  79	sound or menu func.	sound or menu func.	sound or menu func.  80	cursor step up		cursor step up		cursor step up  81	cursor step down	cursor step down	cursor step down  82	menu on			menu on			menu on  83	menu off		menu off		menu off  84	display A/V status	display A/V status	display A/V status  85	cursor step left	cursor step left	cursor step left  86	cursor step right	cursor step right	cursor step right  87	menu function		menu function		menu function  88	PIP on/off		PIP on/off		PIP on/off  89	PIP shift		PIP shift		PIP shift  90	PIP main swap		PIP main swap		PIP main swap  91	strobe on/off		strobe on/off		strobe on/off  92	multi strobe		multi strobe		multi strobe  93	main freezed		main freezed		main freezed  94	3/9 multi scan		3/9 multi scan		3/9 multi scan  95	PIP mode select		PIP mode select		PIP mode select  96	mosaic			mosaic			mosaic  97	solarization		solarization		solarization  98	main stored		main stored		main stored  99	PIP strobe		PIP strobe		PIP strobe 100	recall main		recall main		recall main 101	PIP freeze		PIP freeze		PIP freeze 102	PIP step up		PIP step up		PIP step up 103	PIP step down		PIP step down		PIP step down 104 105 106 107				red 108				green 109				yellow 110				cyan 111				index 112 113 114 115 116 117 118	sub mode		sub mode		sub mode 119	sub mode		sub mode		sub mode 120 121 122	cleaning up		cleaning up		cleaning up 123	connect			connect			connect 124	disconnect		disconnect		disconnect 125	cleaning up		cleaning up		cleaning up 126	cleaning up		cleaning up		cleaning up 127	cleaning up

cleaning up		cleaning up ------------------------------------------------------------------------------  ------------------------------------------------------------------------------ Commnd:	Video mode: LV1		Video mode: VCR1 & VCR2	Video mode: Satellite 	Group 4			Group 5	(VCR1) 6 (VCR2)	Group 8 ------------------------------------------------------------------------------   0	0			0			0   1	1			1			1   2	2			2			2   3	3			3			3   4	4			4			4   5	5			5			5   6	6			6			6   7	7			7			7   8	8			8			8   9	9			9			9  10	picture number/time	1/2 digits/AM/PM	1/2 digits  11	channel number		channel/program mode	channel/program mode  12	standby			standby			standby  13	mute/demute		mute/demute		mute/demute  14	P.P. (personal pref.)	P.P.			P.P.  15	display			display			display  16	volume +		volume +		volume +  17	volume -		volume -		volume -  18	brightness +		brightness +		brightness +  19	brightness -		brightness -		brightness -  20	colour saturation +	colour saturation +	colour saturation +  21	colour saturation -	colour saturation -	colour saturation -  22	bass +			bass +			bass +  23	bass -			bass -			bass -  24	treble +		treble +		treble +  25	treble -		treble -		treble -  26	balance right		balance right		balance right  27	balance left		balance left		balance left  28	repeat once/TSPo	ITR (OTR) delay  29	repeat cont/TSPc	timer programming  30	next			record select  31	fast run reverse	fast run reverse  32	entry			step +			channel/program +  33	auto stop/memory	step -			channel/program -  34	slow run reverse	slow run reverse  35	audio 1			audio output select	language select/mode 1  36	audio 2			clock/channel		special stereo on/off  37	still picture		speed -			audio mixed / mode 2  38	speed +			speed +  39	speed -			Instant Time Recording  40	slow run forward	slow run forward  41	still forward		still forward  42	fast run forward	fast run forward  43	search automatic	index scan  44	scan reverse		scan reverse  45	open/close (eject)	eject  46	scan forward		scan forward		subtitle  47	play reverse		play reverse  48	pause			pause			pause  49	erase			erase			erase  50	rewind			rewind			rewind  51	go to			go to			go to  52	wind			wind			wind  53	play			play			play  54	stop			stop			stop  55	recording		recording		recording  56	external 1		external 1		external 1  57	external 2		external 2		external 2  58	clear all memory	short/long play  59	freeze segment		counter memory  60	TXT (pal)/ant.sw (ntsc)	scroll local display	TXT submode  61	system standby		system standby		system

standby  62	CX on/off		RF switch  63	system select		system select		system select  64	surround sound		surround sound		surround sound  65	balance front		balance front		balance front  66	balance rear		balance rear		balance rear  67	sound effects		sound effects		sound effects  68	sound effects		sound effects		sound effects  69	sound effects		sound effects		sound effects  70	sound effects		sound effects		sound effects  71	sound effects		sound effects		sound effects  72	sound or menu func.	sound or menu func.	sound or menu func.  73	sound or menu func.	sound or menu func.	sound or menu func.  74	sound or menu func.	sound or menu func.	sound or menu func.  75				data stream start  76				data stream end  77	linear function up	linear function up	linear function up  78	linear function down	linear function down	linear function down  79	sound or menu func.	catalogue		sound or menu func.  80	cursor step up		cursor step up		cursor step up  81	cursor step down	cursor step down	cursor step down  82	menu on			menu on			menu on  83	menu off		menu off		menu off  84	display A/V status	display A/V status	display A/V status  85	cursor step left	cursor step left	cursor step left  86	cursor step right	cursor step right	cursor step right  87	menu function		acknowledge		menu function  88	PIP on/off		PIP on/off		PIP on/off  89	PIP shift		PIP shift		PIP shift  90	PIP main swap		PIP main swap		PIP main swap  91	strobe on/off		strobe on/off		strobe on/off  92	multi strobe		multi strobe		multi strobe  93	main freezed		main freezed		main freezed  94	3/9 multi scan		3/9 multi scan		3/9 multi scan  95	PIP mode select		PIP mode select		PIP mode select  96	mosaic			mosaic			mosaic  97	solarization		solarization		solarization  98	main stored		main stored		main stored  99	PIP strobe		PIP strobe		PIP strobe 100	recall main		recall main		recall main 101	PIP freeze		PIP freeze		PIP freeze 102	PIP step up		PIP step up		PIP step up 103	PIP step down		PIP step down		PIP step down 104				automatic start ID 105				write skip ID 106				skip on/off 107				write end ID 108				write FTS 109				write TOC 110 111 112				viss index next 113				viss index previous 114				mark 115				erase 116				renumber 117				blank search 118	sub mode		sub mode		TV1 119	sub mode		sub mode		sub mode 120				auto repeat 121				insert temp 122				scr loc disp 123	connect			connect			connect 124	disconnect 125 126 127 ------------------------------------------------------------------------------  ------------------------------------------------------------------------------ Commnd:	Audio mode: LV2/CDLV	Audio mode: Pre-Amp	Audio mode: Tuner 	Group 12		Group 16 (#1) 19 (#2)	Group 17 ------------------------------------------------------------------------------   0	0			0

0   1	1			1			1   2	2			2			2   3	3			3			3   4	4			4			4   5	5			5			5   6	6			6			6   7	7			7			7   8	8			8			8   9	9			9			9  10	picture number/time	graphic equalizer left	1/2 digits  11	chapter number		graphic equalizer right  12	standby			standby			standby  13	mute/demute		mute/demute		mute/demute  14	P.P. (personal pref.)	P.P.			P.P.  15	display			display			display  16	volume +		volume +		volume +  17	volume -		volume -		volume -  18	brightness +		brightness +		brightness +  19	brightness -		brightness -		brightness -  20	colour saturation +	colour saturation +	colour saturation +  21	colour saturation -	colour saturation -	colour saturation -  22	bass +			bass +			bass +  23	bass -			bass -			bass -  24	treble +		treble +		treble +  25	treble -		treble -		treble -  26	balance right		balance right		balance right  27	balance left		balance left		balance left  28	repeat once/TSPo	graph equal left&right  29	repeat cont/TSPc	loudness  30	next			high tone filter	search +  31	programming		low tone filter		search -  32				step +			preset +  33	auto stop/memory	step -			preset -  34	slow run reverse	signal path scroll  35	stereo left/right  36	previous  37	still reverse		surround mode		mono/stereo  38	speed +			sleeptimer  39	speed -			balance rear  40	slow run forward	balance front		RF switch  41	still forward		TPS			store execute  42	digital multi screen	timer set  43	strobe			timer up		scan forward  44	scan reverse		timer down  45	open/close (eject)	timer memo		FM  46	scan forward		acoustic memory		MW (USA: MW/FM)  47				acoustic memory select	LW  48	pause			pause			pause  49	erase			erase			erase  50	rewind			rewind			rewind  51	go to			go to			go to  52	wind			wind			wind  53	play			play			play  54	stop			stop			stop  55	recording		recording		recording  56	external 1		external 1		external 1  57	external 2		external 2		external 2  58	clear all memory	clear all memory	clear all memory  59	repeat A/B program  60	TXT (pal)/ant.sw (ntsc)	dyn range expand/compr  61	system standby		system standby		system standby  62	CX on/off  63	system select		system select		system select  64	surround sound		surround sound		surround sound  65	balance front		balance front		balance front  66	balance rear		balance rear		balance rear  67	sound effects		sound effects		sound effects  68	sound effects		sound effects		sound effects  69	sound effects		sound effects		sound effects  70	sound effects		sound

effects		sound effects  71	sound effects		sound effects		sound effects  72	sound or menu func.	sound or menu func.	sound or menu func.  73	sound or menu func.	sound or menu func.	sound or menu func.  74	sound or menu func.	sound or menu func.	sound or menu func.  75  76  77	linear function up	linear function up	linear function up  78	linear function down	linear function down	linear function down  79	sound or menu func.	catalogue		sound or menu func.  80	cursor step up		cursor step up		cursor step up  81	cursor step down	cursor step down	cursor step down  82	menu on			menu on			menu on  83	menu off		menu off		menu off  84	display A/V status	display A/V status	display A/V status  85	cursor step left	cursor step left	cursor step left  86	cursor step right	cursor step right	cursor step right  87	menu function		menu function		menu function  88	PIP on/off		PIP on/off		PIP on/off  89	PIP shift		PIP shift		PIP shift  90	PIP main swap		PIP main swap		PIP main swap  91	strobe on/off		strobe on/off		strobe on/off  92	multi strobe		multi strobe		multi strobe  93	main freezed		main freezed		main freezed  94	3/9 multi scan		3/9 multi scan		3/9 multi scan  95	PIP mode select		PIP mode select		PIP mode select  96	mosaic			mosaic			mosaic  97	solarization		solarization		solarization  98	main stored		main stored		main stored  99	PIP strobe		PIP strobe		PIP strobe 100	recall main		recall main		recall main 101	PIP freeze		PIP freeze		PIP freeze 102	PIP step up		PIP step up		PIP step up 103	PIP step down		PIP step down		PIP step down 104 105 106

107

108	FTS

109

110

111

112	index next

113	index previous

114

115

116

117

118	sub mode

119	sub mode

120

121

122

123	connect			connect			connect

124	disconnect		disconnect		disconnect

125

126

127

------------------------------------------------------------------------------

------------------------------------------------------------------------------ Commnd:	Audio mode: CD		Audio mode: Phono	Audio mode: Recorder 	Group 20		Group 21		Group 18 (#1) 23 (#2)

------------------------------------------------------------------------------   0	0			0			0   1	1			1			1   2	2			2			2   3	3			3			3   4	4			4			4   5	5			5			5   6	6			6			6   7	7			7			7   8	8			8			8   9	9			9			9  10	cursor scroll  11	display scroll  12	standby			standby			standby  13	mute/demute		mute/demute		mute/demute  14	P.P. (personal pref.)	P.P.			P.P.  15	display			display			display  16	volume +		volume +		volume +  17	volume -		volume -		volume -  18	brightness +		brightness +		brightness +  19	brightness -		brightness -		brightness -  20	colour saturation +	colour saturation +	colour saturation +  21	colour saturation -	colour saturation -	colour saturation -  22	bass +			bass +			bass +  23	bass -			bass -			bass -  24	treble +		treble +		treble +  25	treble -		treble -		treble -  26	balance right		balance right		balance right  27	balance left		balance left		balance left  28	rept once/TSPo/shuffle	repeat once		repeat once  29	repeat cont/TSPc	repeat continuously	repeat continuously  30	select +		select +		select +  31	select -		select -		select -  32	next			next			next  33	previous		previous		previous  34	index next		index next  35	index previous		index previous		last record cancel  36	play/program		stereo/mono		sequential  37	speed nominal					blank search  38	speed +  39	speed -  40							recording pause  41	store execute		store execute		store execute  42							recording blank  43	scan forward		scan forward		scan forward  44							mechanism A  45	open/close		open/close		eject  46				wave scroll		mechanism B  47							play reverse  48	pause			pause			pause  49	erase			erase			erase  50	rewind			rewind			rewind  51	go to			go to			go to  52	wind			wind			wind  53	play			play			play  54	stop			stop			stop  55	recording		recording		recording  56	external 1		external 1		external 1  57	external 2		external 2		external 2  58	clear all memory	clear all memory	clear all memory  59	repeat A/B program  60	dynamic range expand  61	system standby		system standby		system standby  62	dynamic range compress				counter memory  63	system select		system select		system select  64	surround sound		surround sound		surround sound  65	balance front		balance front		balance front  66	balance

rear		balance rear		balance rear  67	sound effects		sound effects		sound effects  68	sound effects		sound effects		sound effects  69	sound effects		sound effects		sound effects  70	sound effects		sound effects		sound effects  71	sound effects		sound effects		sound effects  72	sound or menu func.	sound or menu func.	sound or menu func.  73	sound or menu func.	sound or menu func.	sound or menu func.  74	sound or menu func.	sound or menu func.	sound or menu func.  75  76  77	linear function up	linear function up	linear function up  78	linear function down	linear function down	linear function down  79	sound or menu func.	sound or menu func.	sound or menu func.  80	cursor step up		cursor step up		cursor step up  81	cursor step down	cursor step down	cursor step down  82	menu on			menu on			menu on  83	menu off		menu off		menu off  84	display A/V status	display A/V status	display A/V status  85	cursor step left	cursor step left	cursor step left  86	cursor step right	cursor step right	cursor step right  87	menu function		menu function		menu function  88	PIP on/off		PIP on/off		PIP on/off  89	PIP shift		PIP shift		PIP shift  90	PIP main swap		PIP main swap		PIP main swap  91	strobe on/off		strobe on/off		strobe on/off  92	multi strobe		multi strobe		multi strobe  93	main freezed		main freezed		main freezed  94	3/9 multi scan		3/9 multi scan		3/9 multi scan  95	PIP mode select		PIP mode select		PIP mode select  96	mosaic			mosaic			mosaic  97	solarization		solarization		solarization  98	main stored		main stored		main stored  99	PIP strobe		PIP strobe		PIP strobe 100	recall main		recall main		recall main 101	PIP freeze		PIP freeze		PIP freeze 102	PIP step up		PIP step up		PIP step up 103	PIP step down		PIP step down		PIP step down 104							automatic start ID 105							write skip ID 106							skip on/off 107							write end ID 108	FTS						write FTS 109							write TOC 110 111 112 113 114							write start ID 115							Ca/Er/SK ID 116							reformat 117							blank search 118	sub mode		sub mode		sub mode 119	sub mode		sub mode		sub mode 120 121 122 123	connect			connect			connect 124	disconnect		disconnect		disconnect 125 126 127

------------------------------------------------------------------------------

The codes and parts of the RC5REMOTE program are copied from the article "Remote Control for HP28S" written by Michiel Niemeijer & Eric Toonen.